|  |  |  |  |  |  |  |  | # Page 1 of 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # PARTS LIST |  |  |  |  |  |  |  |  |  |  |  |
| PROJECT: | # PLL Small Hydro Controller |  |  |  |  |  | VERSION: | # 12.1 |  | DATE: | # June 12, 2007 |
| ### PART I.D. |  | # PART DESCRIPTION |  | #/UNIT | # SOURCE INFO/NOTES |  |  |  |  |  |  |
| # C1 |  | 47 uF 25V |  | 1 | Digi-Key #P6238-ND |  |  |  |  |  |  |
| C2 |  | 47 uF 10V |  | 1 | Digi-Key #P6213-ND |  |  |  |  |  |  |
| C3 |  | 10 uF 25V tantalum |  | 1 | Digi-Key 478-1841-ND |  |  |  |  |  |  |
| C4, C14 |  | .022 uF ceramic |  | 3 | Digi-Key BC1080CT-ND (10 for 90 cents) |  |  |  |  |  |  |
| C5, C6 |  | 1 uF electrolytic |  | 2 | Digi-Key P6260-ND |  |  |  |  |  |  |
| C7, C8 |  | 18 pF ceramic |  | 2 | Digi-Key 490-3697-ND |  |  |  |  |  |  |
| C9-12 |  | 10 uF 25V tantalum |  | 4 | Digi-Key 478-1841-ND |  |  |  |  |  |  |
| C15 |  | 1000 uF 25V |  | 1 | Digi-Key P5156-ND |  |  |  |  |  |  |
| #### D1-3, 10, 11, 13, 15,16 |  | 1N4003 |  | 8 | Digi-Key 1N4003-E3/54GICT-ND |  |  |  |  |  |  |
| #### D4 |  | Yellow T1 LED, Right Angle Mount |  | 1 | Digi-Key 350-1624-ND (Dialight 550-1207) - substitution may cause poor fit! |  |  |  |  |  |  |
| D5 |  | Green T1 LED, Right Angle Mount |  | 1 | Digi-Key 350-1627-ND (Dialight 550-1307) - ditto |  |  |  |  |  |  |
| D6 |  | Red T1 LED, Right Angle Mount |  | 2 | Digi-Key 350-1620-ND Dialight 550-1107 - ditto - includes 1 extra, use mount for D7 |  |  |  |  |  |  |
| D7 |  | White T1 LED, Right Angle Mount |  | 1 | Digi-Key CMD333UWC-ND with base ridge ground off to fit above mount |  |  |  |  |  |  |
| D8 |  | Blue***T1 LED, Right Angle Mount |  | 1 | Digi-key 350-1843-ND (Dialight 550-1007) |  |  |  |  |  |  |
| D9,12,14,17,18 |  | 6 amp 100 PIV rectifier |  | 3 or 5 | MR754RLGOSCT-ND (5 incl. back-offs on water control jackscrew assy) |  |  |  |  |  |  |
| F1 |  | Fuse, 8 amp (in Battery Cable) |  | 1 | Automotive, obtain fuse and an in-line fuseholder locally. |  |  |  |  |  |  |
| F2 |  | Fuse ½ amp (Load Management Line) |  | 1 | Digi-Key 283-2620-ND, obtain an in-line automotive fuseholder locally. |  |  |  |  |  |  |
| GND LUG |  | Keystone #1287 Spade Lug |  | 1 | Carson Electronics has lots 06/2007 |  |  |  |  |  |  |
| HD1, 2, 5 |  | Headers 7 to 20 pin X 2 X 0.1” |  | 3 | Digi-Key A26529-40-ND (snap off at the right length) |  |  |  |  |  |  |
| ##### HD3&4, TP1-2, JP1 |  | Headers 1 X 1 to 1 X 6 X 0.1” |  | 3 | Digi-Key 929450-01-36-ND (snap off at the right length) |  |  |  |  |  |  |
| ##### J1 |  | DB9 Female Right Angle PCB Conn. |  | 1 | Mode 30-420-0 from Electrosonic. Must be 8.1mm front board holes to face. |  |  |  |  |  |  |
| K1 |  | Relay 12V DPDT |  | 1* | Digi-Key PB771-ND (Potter & Brumfield R10-E1W2-V185) |  |  |  |  |  |  |
| L1 |  | RF Choke 1500 uH RCD |  | 1 | RCD type AL05-1.5K (max 60 ma - probably better to use 10 ohm resistor) |  |  |  |  |  |  |
| M1 |  | 12V Wiper Motor |  | 1* | Ford PU 1970's type - extremely reliable. Un-ground neg. brush before use!!! |  |  |  |  |  |  |
| Q1, Q3 |  | 2N2222A NPN (TO-92) |  | 2 (1*) | Digi-Key PN2222BU-ND |  |  |  |  |  |  |
| Q2 |  | ZTX792A load mgmt. drive transistor |  | 1 | Digi-Key ZTX792A-ND |  |  |  |  |  |  |
| Q4, 5, 6, 7 |  | Power Mosfet, Logic Level In |  | 1 | Digi-Key IRLI3705NPBF-ND |  |  |  |  |  |  |
| #### R1,6-8,12,18,19,35,36 |  | 12K ohms |  | 12 | Digi-Key 12KQBK-ND a penny each if you buy 1000, 31 pennies for 5 |  |  |  |  |  |  |
| ###### R2 |  | 4.7K ohms |  | 1 | Digi-Key 4.7KQBK-ND - ditto |  |  |  |  |  |  |
| R3, R25 |  | 47K ohms |  | 2 | Digi-Key 47KQBK-ND |  |  |  |  |  |  |
| # R4, 26-34 |  | 1K ohms |  | 10 | Digi-Key 1.0KQBK-ND |  |  |  |  |  |  |
| R5 |  | 10M ohms |  | 1 | Digi-Key 10MQBK-ND |  |  |  |  |  |  |
| R9 |  | 10K ohms trimpot W/Panel Mount |  | 1 | Digi-Key 3006P-103LF-ND pot. Carson Electronics has necessary H083P mount. |  |  |  |  |  |  |
| R20-24 |  | 470 ohms 330 ohms (180 ohms for 15 ma optos) |  | 5 | Digi-Key 470QBK-ND |  |  |  |  |  |  |
| R37, 38, 39, 40 |  | 1.5K ohms (R40 goes in C13’s place). |  | 4 (2*) | Digi-Key 1.5KQBK-ND |  |  |  |  |  |  |
| S1 |  | SPST N.O. Keyswitch Right Angle |  | 1 | Digi-Key SW409-ND (Omron B3F-3150) |  |  |  |  |  |  |
| S2 |  | Toggle Switch, R. Angle PCB mount |  | 1 | Digi-Key 450-1386-ND (Alco TT11DG-VRA-104) |  |  |  |  |  |  |
| S3 |  | Toggle Switch, heavy, SPDT |  | 1* | Pollak 34-578Q (center off, spring return one way, stays put other way). |  |  |  |  |  |  |
| T1 |  | Power Transformer, Flat |  | 1 | Digi-Key 229A16-ND |  |  |  |  |  |  |
| TB1 |  | 4 Pin Header W/Screw Term. Plug |  | 1 | Digi-Key WM5848-ND (mates with Digi-Key WM5829-ND - get both) |  |  |  |  |  |  |
| TB2 |  | 10 Pin Header W/Screw Term. Plug |  | 1 | Digi-Key WM5852-ND (mates with Digi-Key WM5830-ND - get both) |  |  |  |  |  |  |
| TB3 |  | 5 Pin Header W/Screw Term. Plug |  | 1 | Digi-Key WM5849-ND (mates with Digi-Key WM5833-ND - get both) |  |  |  |  |  |  |
| U1 |  | HC11 Microcontroller E2/E9 |  | 1 | Carson Electronics has secured and unsecured ones. |  |  |  |  |  |  |
| U2 |  | 74LS132 Schmitt Quad NAND |  | 1 | Digi-Key 296-3645-5-ND |  |  |  |  |  |  |
| U3 |  | 7805 Regulator |  | 1 | Digi-Key LM7805CD-ND |  |  |  |  |  |  |
| U4 |  | Seiko S-8054HN Power Monitor |  | 1 | Carson Electronics has >100 of these |  |  |  |  |  |  |
| U5 |  | LT1081CN RS232 Chip |  | 1 | Digi-Key LT1081CN#PBF-ND. I have had no luck with Dallas Semi ones. |  |  |  |  |  |  |
| U6-U10 |  | MOC3083 Triac Opto (& see page 3) |  | 5 | Digi-Key MOC 3083M-ND (not needed if Zero-crossing SSR's used) |  |  |  |  |  |  |
| U11, U12 |  | 74HC595 Shift Register for SPI Bus |  | 2 | Digi-Key MM74HC595N-ND |  |  |  |  |  |  |
| Y1 |  | 8 MHz Crystal |  | 1 | Digi-key X1093-ND (not as tall as standard, good for LMD, main board too) |  |  |  |  |  |  |

C:\Art\Company\vptParts List-VPT3_1.doc   *for actuator and its driver   **for experimental solid state H-bridge actuator reversing switch  ***Blue phase LED is a  "brand" of the unit - don't change!  Rev. 0.0  June 12, 2007

|  |  |  |  |  |  |  |  | # Page 2 of 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # PARTS LIST |  |  |  |  |  |  |  |  |  |  |  |
| PROJECT: |  | # PLL Small Hydro Controller |  |  |  |  | VERSION: | # 12.1 |  | DATE: | # June 12, 2007 |
| ### PART I.D. | # PART DESCRIPTION |  |  | #/UNIT | # SOURCE INFO/NOTES |  |  |  |  |  |  |
|  | VPT12.1 Circuit Board |  |  | 1 | Carson Electronics has a few. Mfg. data at www.carsons.ca/hydmania.htm |  |  |  |  |  |  |
|  | 52 Pin PLCC Socket |  |  | 1 | Digi-Key ED80004-ND |  |  |  |  |  |  |
|  | Fuse Socket, In-line |  |  | 2 | Get at any automotive supply store |  |  |  |  |  |  |
|  | #4-40 Screw, ¾” long |  |  | 6 | 4-40 screws can be found in most electronics catalogues. |  |  |  |  |  |  |
|  | #4-40 nut |  |  | 6 | Ditto. |  |  |  |  |  |  |
|  | 3/16-24 screw, ½” |  |  | varies | 3/16" screws are found in (or can be ordered by) harware stores. |  |  |  |  |  |  |
|  | 3/16-24 screw, 1 ½” |  |  | varies | The 3/16" screws go into tapped holes. No nuts. |  |  |  |  |  |  |
|  | Relay Hold Down Spring |  |  | 1* | Digi-Key PB136-ND (Potter & Brumfield 20C249) |  |  |  |  |  |  |
|  | Relay Socket, Right Angle |  |  | 1* | Digi-Key PB808-ND (P&B 27E317) -doubles as mount for driver electronics |  |  |  |  |  |  |
|  | 8 Amp 600 or 800 Volt Triac |  |  | 1 | Suggest using solid state switch instead - 3-20 volts in, 10A 240V out |  |  |  |  |  |  |
|  | 15 Amp 600 or 800 Volt Triac |  |  | 2 | Suggest using solid state switch instead - 3-20 volts in, 25A 240V out |  |  |  |  |  |  |
|  | 40 Amp 600 or 800 Volt Triac |  |  | 2/3/6 | Suggest using solid state switch instead - 3-20 volts in, 60A 240V out |  |  |  |  |  |  |
|  | Aluminum Tank, 4, 5, 6, 9, or 10 Port |  |  | 1 | Carson Electronics has 3" X ½" and 3" X ¾" bar stock for sides, top, bottom |  |  |  |  |  |  |
| LCD1 | 2X20 Character LCD Module |  |  | 1 | Digi-Key 67-1762-ND. Run backlight off unreg. supply. Use resistor to give backlight <200ma. |  |  |  |  |  |  |
| PL3, PL4 | 2 X 7 X 0.1” Header plugs for LCD |  |  | 2 | Digi-Key CSC14G-ND |  |  |  |  |  |  |
|  | 14 Cond. Flat Cable for LCD |  |  | 1 | Can rip 14-wide from old computer cables. length about 4½ to 5 inches. |  |  |  |  |  |  |
|  | 2X20 Header Plug for System Bus |  |  | 1 | Digi-Key CSC40G-ND |  |  |  |  |  |  |
|  | Polycarbonate Window for LCD |  |  | 1 | Carson Electronics has a few. |  |  |  |  |  |  |
|  | Panel Graphics Label Set |  |  | 1 | Included in construction info. Laser print on transparent adhesive-back sheet. |  |  |  |  |  |  |
|  | Front Panel |  |  | 1 | Indexer motion figures updated since last mfr. See literature. made from old circuit board stock. |  |  |  |  |  |  |
|  | Tank Foot Aluminum Spacer |  |  | 3 | Holesaw cutouts. Drill 2 small holes to bolt to tank, use centre hole for wall mounting. |  |  |  |  |  |  |
|  | Spray Lacquer, Clear |  |  | 1/8 | I.E. Krylon - for coating label |  |  |  |  |  |  |
|  | Recreational White Spray Paint |  |  | ¼ can | For finishing tank. mask off any SSR or semiconductor mount areas etc. |  |  |  |  |  |  |
|  | ½ NPT Electrical Connector |  |  | 3 | 2-screw clamp type available from hardware stores |  |  |  |  |  |  |
|  | ¾ or 1¼ NPT Electrical Connector |  |  | 1 | ditto |  |  |  |  |  |  |
|  | 5/16” X ¾” Bolt |  |  | 24/40 | hardware store item |  |  |  |  |  |  |
|  | 5/16” Flat Washer |  |  | 24/40 | ditto |  |  |  |  |  |  |
|  | 5/16” Lock Washer |  |  | 24/40 | ditto |  |  |  |  |  |  |
|  | Heater Bolt Insulating Sleeve 5/16” |  |  | 20/36 | Cut from plastic sleeving. Insulates heaters from tank for dielectric protection |  |  |  |  |  |  |
|  | Heater Bolt Insulating Washer 5/16” |  |  | 20/36 | Cut from gasket material with 2 sizes of punch. Insulates heaters as above. |  |  |  |  |  |  |
|  | Heater Gasket with 4 bolt holes |  |  | 4 to10 | Make up or use the ones that come with the heating elements. |  |  |  |  |  |  |
|  | Cleanout Plate, Aluminum |  |  | 1 | Make from scrap of aluminum plate. Has 4 bolts and covers holesawed hole. |  |  |  |  |  |  |
|  | Side Cover, Aluminum |  |  | 1 or 2 | Same type of plate as used for main front & back pieces. |  |  |  |  |  |  |
|  | Top Cover, Aluminum |  |  | 1 | Ditto. |  |  |  |  |  |  |
|  | Front and back of bolted-type tank |  |  | 1 or 2 | 3/16" aluminum plate (can use 1/8 if need be) |  |  |  |  |  |  |
|  | Conformal Coating |  |  | can | Used to coat circuit board to resist dampness. Digi-Key has this as spray. |  |  |  |  |  |  |
|  | Aluminum Tapping Compound, Relton A9 |  |  | 1/50 | For drilling & tapping aluminum. Made from animal fat. Kitchen lard works. |  |  |  |  |  |  |
|  | 63/37 Solder, Electronics |  |  | spool | Digi-Key KE-1103-ND (Kester 24-6337-0010) is a good choice (not pb free) |  |  |  |  |  |  |
|  | Heat Sink Compound (Silicone Grease) |  |  | 1/10 | Place under solid state relays, tab-mounted transistors etc. Absolutely necessary for cooling. |  |  |  |  |  |  |
|  | Angle Bracket, Board to Panel |  |  | 2 | These are cut from approx. 1/16 inch aluminum, see photos. |  |  |  |  |  |  |
|  | #6 Split Bolt Connector |  |  | 2 or 3 | For connecting main supply to bundles of heater wires. Insulate with tape. |  |  |  |  |  |  |
|  | Tank Ground Terminal |  |  | 1 | For grounding main supply cable to tank. Available in hardware stores. |  |  |  |  |  |  |
|  | #12 & 10 - 1 Cond. Cable for Heaters |  |  | 5-8 ft | Solid wires, can strip out of ordinary NMD-90 building cable. |  |  |  |  |  |  |
|  | PVC Slip to ½” NPT Male Adapter |  |  | 2 | Used for dielectric protection on metallic in & out water pipes to tank. |  |  |  |  |  |  |
|  | Firmware, Version 7.3X or later |  |  | 1 | Available free at www.carsons.ca/mtnmania.htm |  |  |  |  |  |  |
|  | 750W 240V Heater |  |  | 1 or 3 | Carson Electronics has quite a few - 240V |  |  |  |  |  |  |
|  | 1500W 240V Heater |  |  | 1 or 2 | Readily available for domestic element replacement |  |  |  |  |  |  |
|  | 3000W 240V Heater |  |  | 1 | Ditto. See 240/120V note below, which applies to all these heater sizes. |  |  |  |  |  |  |
|  | 6000W 240VHeater |  |  | 1,3,6 | May require special order. (use all these 4 sizes of 240V elements at 120V to cut power 4:1). |  |  |  |  |  |  |
|  | Water Filter |  |  | 1 | Domestic whole-house type from hardware store; drill 1/16" holes in filter |  |  |  |  |  |  |

C:\Art\Company\ptc\Parts List-VPT3_1.doc			*for actuator and its driver							          Rev. 0.0  June 12, 2007

|  |  |  |  |  |  |  |  | # Page 3 of 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # PARTS LIST |  |  |  |  |  |  |  |  |  |  |  |
| PROJECT: |  | # PLL Small Hydro Controller |  |  |  |  | VERSION: | # 12.1 |  | DATE: | # June 12, 2007 |
| ### PART I.D. | # PART DESCRIPTION |  |  | #/UNIT | # SOURCE INFO/NOTES |  |  |  |  |  |  |
|  | Screen for Water Filter |  |  | 1 | can use in place of stock domestic filter element which is unnecessarily fine. |  |  |  |  |  |  |
|  | Load Mgmt. Line Lightning Zener |  |  | 1 | Digi-key 1.5KE18AG1CT-ND (18 volt single-direction. Both ends of line.) |  |  |  |  |  |  |
|  | RS232 “ “ , Bi-dir, 1.5KE15CAG1CT |  |  | 2 | Digi-key 1.5 KE15CAG1CT-ND (15 volt bi-directional, for long serial cable) |  |  |  |  |  |  |
|  | DC Tank Terminal Board, 8 Screw |  |  | 1 | Essco Flex Block #4EDS is one option. |  |  |  |  |  |  |
|  | RS232 Chip Socket, 16 pin DIP |  |  | 1 | If any chip is going to get zapped it's the serial port one, so put it in a socket. |  |  |  |  |  |  |
| U101 | Heat Sensor, LM56CIM |  |  | 1 | These are used for the vital tank overtemp trip. Wire up as in main schematic. |  |  |  |  |  |  |
|  | Brass Fitting for Potting the LM56 in |  |  | 1 | Weatherhead 100X5. Fill with Bondo resin. See photo for convenient jig. |  |  |  |  |  |  |
|  | LED Base Ridge Removal Tool (hose) |  |  | 1 | 2-inch piece of rubber hose, I.D. a bit less then LED diam, to hold LED D7 in hand for grinding. |  |  |  |  |  |  |
|  | DB9 Plug and Shell for Computer Cable |  |  | 1 | Male: Digi-Key 209M-ND. Female: see catalog. Shell is 909GP-ND. |  |  |  |  |  |  |
|  | Plastic Tie Anchor, Stick-on |  |  | 8 | Varous types exist. I am finding that the glue tends to fail eventially. |  |  |  |  |  |  |
|  | Test Lead Wire for High V., Low Current |  |  | ~12 ft | DABURN 2722/22 in red, black, yellow, green. Great 5000V hookup wire! |  |  |  |  |  |  |
|  | Resistors for running Opto’s off 240V |  |  | 10** | Calculate, specify and test values for opto's on 3-phase boards before use. |  |  |  |  |  |  |
|  | 3 Phase Adapter Board |  |  | 1** | 2 sets of extra zero-crossing opto's on perf board for 2 extra phases. Not req'd with o-xing SSRs. |  |  |  |  |  |  |
|  | MOC3083 Opto’s for 3 Phase Adapter |  |  | 10** | Place on perf and connect to auxiliary opto drive header connector. See notes for U6-U10. |  |  |  |  |  |  |
|  | 3/16” Lock Washer |  |  |  | Standard practice to use under bolt heads throughout tank unit. |  |  |  |  |  |  |
|  | 3/16” Nut |  |  |  | May be needed for any non-tapped holes; very few if any needed. |  |  |  |  |  |  |
|  | Large Marrette for 2.5 KW Tank |  |  | 1 | In smaller controllers, large marrettes may be used instead of split-bolt conn. |  |  |  |  |  |  |
|  | Aluminum Angle/square for Tank Corner |  |  | 1 | Not needed in current tank design; uses bent tabs on AC/DC cable panels. |  |  |  |  |  |  |
| R101,4,5 | 18.2K ohms 1% (For Thermostat) |  |  | 3 | Digi-Key 18.2KXBK-ND |  |  |  |  |  |  |
| R102 | 1.5K ohms 1% (For Thermostat) |  |  | 1 | Digi-Key 1.50KXBK-ND |  |  |  |  |  |  |
| R103 | 10K ohms 1% (For Thermostat) |  |  | 1 | Digi-Key 10.0KXBK-ND |  |  |  |  |  |  |
|  | * not needed if no water control module |  |  |  |  |  |  |  |  |  |  |
|  | ** not needed if no 3-phase adapter |  |  |  |  |  |  |  |  |  |  |
|  | rev.1 Sept. 16/2010 (original date still in heading) |  |  |  | removed reference to 4 MOSFETs for relayless motor drive (idea not pursued) |  |  |  |  |  |  |

C:\Art\Company\Parts List-VPT3_1.doc				**for three phase adapter						          Rev. 0.0  June 12, 2007